Argos Froyanes Gear details. Line type; Autoline. Line material; 12 mm thick Polyester/Nylon braided rope. Average line length and range 6,708 m (4,500 – 10,800). Hook type; 14/0 curved shank. Number of hooks per line and spacing2; 4,472 (3,000 – 7,200) at 1.6 m spacing. Snood Length; 0.4m. Weight material and mass; Integrated weight, 50 g/m. Spacing of weights; N/A. Sink rate 0.37m/s. Anchor type and number; One 65/80kg grapnel anchor and one 40/45kg chain at either end.. Floats and spacing; N/A. The vessel uses two different gear configurations depending on the topography of the fishing grounds and the ice conditions, single lines and join lines, as illustrated above. The hook types and dimensions for the vessel are given below. Type; Curved shank. Make; Fiskevegn. Total length; 60. Shank (mm); 38. Gape (mm); 25. Throat (mm); 32. Front length (mm); 35. Usual setting position. Line off bottom (m); 0. Hooks off bottom (m); 0.

Based on 2015/16 data

For regular gear, measurement of sink rates is no longer required, data are from 2013/15 season.